Tokenless Dining Management System Challenge

Team Name: One_Zero
Date: November 2025

## 1. Introduction

The Tokenless Dining Management System automates dining operations without physical tokens. It uses ID/PIN authentication, RFID card access, and presence detection to manage access, ensure security, and save power.

Key Features: - Multi-user PIN system
- RFID-based card authentication
- IR sensor for presence detection & deep sleep
- LED indications for access

Image placeholder: Optional background image of a cafeteria or ESP32 board.

## 2. System Components

- ESP32 / Arduino

- 2x2 Keypad for PIN entry

- MFRC522 / RFID Reader

- IR Sensor (PIR)

- LEDs for notifications

- OLED/TFT Display (optional)

- Power supply

Image placeholder: Block diagram of ESP32 with arrows to all connected components.

## 3. Module 1: Multi-Person PIN System

#include <Keypad.h>

#define LED_PIN 4
#define LIGHT_TIME 3000

const byte ROWS = 2;
const byte COLS = 2;

char keys[ROWS][COLS] = {{'1','2'},{'4','5'}};
byte rowPins[ROWS] = {12, 13};
byte colPins[COLS] = {14, 15};

Keypad keypad = Keypad(makeKeymap(keys), rowPins, colPins, ROWS, COLS);

String passPiyal = "1245";
String passOrpon = "2514";
String passNihal = "4152";

String input = "";

void resetInput() { input=""; }

void setup() {
  Serial.begin(115200);
  pinMode(LED_PIN, OUTPUT);
  digitalWrite(LED_PIN, LOW);
  Serial.println("=== MULTI PERSON PASSWORD SYSTEM ===");
  Serial.println("Enter 4 keys...");
}

void loop() {
  char key = keypad.getKey();

  if (key) {
    Serial.print("*");
    input += key;

    if (key!='1' && key!='2' && key!='4' && key!='5') {
      Serial.println("\n[X] Wrong Key! Resetting...");
      resetInput();
      return;
    }

    if (input.length() == 4) {
      Serial.println();
      if (input == passPiyal) {
        Serial.println("[OK] Piyal Access Granted");
      } else if (input == passOrpon) {
        Serial.println("[OK] Orpon Access Granted");
      } else if (input == passNihal) {
        Serial.println("[OK] Nihal Access Granted");
      } else {
        Serial.println("[X] Wrong Password! Enter your right Password:");
        resetInput();
        return;
      }
      digitalWrite(LED_PIN, HIGH);
      delay(LIGHT_TIME);
      digitalWrite(LED_PIN, LOW);
      resetInput();
    }
  }
  delay(20);
}

Image placeholder:

## 4. Module 2: IR Sensor with ESP32-CAM & Deep Sleep

#include "esp_camera.h"
#include "driver/rtc_io.h"

#define SENSOR_PIN GPIO_NUM_14
#define LED_PIN    GPIO_NUM_4
#define IDLE_TIMEOUT_MS 5000

#define SENSOR_ACTIVE_LEVEL LOW
#define SENSOR_IDLE_LEVEL HIGH

unsigned long lastActivityTime = 0;

void setup() {
  Serial.begin(115200);
  gpio_hold_dis(LED_PIN);
  gpio_deep_sleep_hold_dis();
  pinMode(LED_PIN, OUTPUT);
  pinMode(SENSOR_PIN, INPUT_PULLUP);
  digitalWrite(LED_PIN, HIGH);
  lastActivityTime = millis();

  esp_sleep_wakeup_cause_t cause = esp_sleep_get_wakeup_cause();
  if (cause == ESP_SLEEP_WAKEUP_EXT0) {
    Serial.println(">>> WOKE UP via IR INTERRUPT! <<<");
  } else {
    Serial.println(">>> Normal Power On <<<");
  }
}

void loop() {
  int sensorState = digitalRead(SENSOR_PIN);

  if (sensorState == SENSOR_ACTIVE_LEVEL) {
    lastActivityTime = millis();
    digitalWrite(LED_PIN, HIGH);
    Serial.println("Motion Detected: Active");
  } else {
    unsigned long currentMillis = millis();
    if (currentMillis - lastActivityTime >= IDLE_TIMEOUT_MS) {
      enterSleepMode();
    }
  }
  
  delay(50);
}

void enterSleepMode() {
  Serial.println("Idle timeout reached. Engaging Interrupt System & Sleep...");
  digitalWrite(LED_PIN, LOW);
  int wakeLogic = (SENSOR_ACTIVE_LEVEL == LOW) ? 0 : 1;
  if (rtc_gpio_is_valid_gpio(SENSOR_PIN)) {
    esp_sleep_enable_ext0_wakeup(SENSOR_PIN, wakeLogic);
    Serial.println("Interrupt Configured on Pin 14.");
  }
  gpio_hold_en(LED_PIN);
  Serial.println("Sleeping...");
  Serial.flush();
  esp_deep_sleep_start();
}

Image placeholder:

## 5. Module 3: RFID Access System

#include <SPI.h>
#include <MFRC522.h>

#define SS_PIN 2
#define RST_PIN 12
#define LED_PIN 4
#define LED_TIME 4000

MFRC522 rfid(SS_PIN, RST_PIN);

String uid_Piyal = "61:EE:EE:00";
String uid_Nihal = "37:79:0E:05";

void setup() {
  Serial.begin(115200);
  SPI.begin(14, 13, 15);
  rfid.PCD_Init();
  pinMode(LED_PIN, OUTPUT);
  digitalWrite(LED_PIN, LOW);
  Serial.println("=== RFID Access System with LED ===");
  Serial.println("Scan your card...");
}

void loop() {
  if (!rfid.PICC_IsNewCardPresent()) return;
  if (!rfid.PICC_ReadCardSerial()) return;

  String UID = "";
  for (byte i = 0; i < rfid.uid.size; i++) {
    if (rfid.uid.uidByte[i] < 0x10) UID += "0";
    UID += String(rfid.uid.uidByte[i], HEX);
    if (i < rfid.uid.size - 1) UID += ":";
  }
  UID.toUpperCase();

  Serial.print("Card UID: ");
  Serial.println(UID);

  if (UID == uid_Piyal) {
    Serial.println("[OK] Piyal Access Granted");
    digitalWrite(LED_PIN, HIGH);
    delay(LED_TIME);
    digitalWrite(LED_PIN, LOW);
  }
  else if (UID == uid_Nihal) {
    Serial.println("[OK] Nihal Access Granted");
    digitalWrite(LED_PIN, HIGH);
    delay(LED_TIME);
    digitalWrite(LED_PIN, LOW);
  }
  else {
    Serial.println("[X] Access Denied (Unknown Card)");
  }

  rfid.PICC_HaltA();
}

Image placeholder:

## 6. Workflow

- IR sensor detects a person → ESP32 wakes up

- User enters PIN or scans RFID card

- Access is granted if PIN/UID matches

- LED feedback is shown for authorized access

- No activity → system enters deep sleep

Image placeholder: Workflow flowchart diagram.